ZD,

REPORT OF JOINT INVESTIGATION VISIT BY NOSDRA, DPR, SPDC, RIVERS STATE MINISTRY OF ENVIRONMENT AND BODO WEST COMMUNITY REPRESENTATIVES AT 28” BOMU – BONNY TRANS NIGER PIPE LINE (TNP) RIGHT OF WAY @ BODO WEST IN GOKANA L.G.A OF RIVERS STATE.

INTRODUCTION

Acting on your directives on the SPDC request latter sent in on 14th September 2015, a Joint Investigation Visit (JIV) was conducted at the 28’’ Bomu – Bonny Trans Niger Pipeline (TNP) Right of Way at Bodo West in Gokana L.G.A of Rivers State on the 13th of September, 2015 at about 12:50Hrs. The investigation was completed successfully on the same day. This report provides details.

INCIDENT PARTICULARS: 		2015_ 1483495

FACILITY AFFECTED:			28’’ BOMU-BONNY TNP ROW @ BODO WEST IN RIVERS

STATE

DATE OF SPILL:				10/09/2015

DATE OF INSPECTION:			13/09/2015

CAUSE OF SPILL:			SABOTAGE (See Fig.1a & b)

QUANTITY OF CRUDE OIL SPILLED:	6.25bbls (Approximately 7.0bbls)

SPILL POINT COORDINATES:		NOTHINGS:  4.58362 (Latitude)

EASTHINGS:  7.25604 (Longitude)

NATURE OF IMPACT:			Area Saturated with Crude Oil

INVESTIGATION TEAM:

- Koms Abam 			NOSDRA

- Chira Nwuzor			SPDC

- Okerinde Solomon 		SPDC

- Opara R. N.			DPR

- Charles Odum			RIVERS STATE MINISTRY OF ENVIRONMENT

- Dennis Kaakpo			BODO WEST COMMUNITY

- Baba Eigege                                    POLICE

OBSERVATIONS:

- Unknown persons installed a 4’’ Nipple with a Ball Valve on the 28’’ Pipeline at 12 O’clock position, for Crude Oil theft. (See Fig. 1a & b)

- The impacted Area is a Tidal Environment with 10 Mud Flats along ROW (See Fig. 2a & b)

- The whole terrain, made up of Alluvial Deposits (mostly Silty and Sandy Clay in texture, with decomposed Plant Roots) has been saturated with Crude Oil. Pool of Free Phase Oil was observed around the Spill Point and since investigation was carried out at Low Tides, most of the Crude Oil must have been carried away by water and infiltration must have also occurred considering the nature of the Topography. (See Fig. 2a & b)

IMPACTED AREA COORDINATES:	1. NOTHINGS:   4.58367		2. NOTHINGS:   4.58362

EASTHINGS:  7.25610 	    EASTHINGS:  7.25610

3. NOTHINGS:   4.58359		4. NOTHINGS:   4.58361

EASTHINGS:  7.25606 	    EASTHINGS:  7.25603

5. NOTHINGS:   4.58369		6. NOTHINGS:   4.58369

EASTHINGS:  7.25606 	    EASTHINGS:  7.25603

- The Vegetation around the impacted Area showed a wide span of harvested White Mangrove (See Fig. 3a & b)

RECOMMENDATIONS

- Surveillance of facilities need to be intensified to prevent future re-occurrence

- The affected area should be completely repaired and immediately cleaned up.

CONCLUSION

The JIV was successful. This report is subject to your steer and further directives.

Koms Abam

SES

_____________________________

14th September, 2015

`

Fig. 1a 4’’ Nipple & Valve installed		   Fig.1b Installed @ 12 O’clock with an Elbow

Fig. 2a ROW with 10 Mud Flats		                 Fig. 2b Alluvial Deposits

Fig. 3a Vegetation around the Spill Area		     Fig. 3b Harvested White Mangrove